# Hybrid Boot Experiment Reference (J721E EVM on BeagleBone AI-64)

## Objective

Boot the TI J721E EVM software stack on the BeagleBone AI-64 by keeping the EVM software stack and changing only the hardware-specific components required for AI-64.

## Main Source Files Investigated

- arch/arm/mach-k3/... : board_init_f()
- drivers/ram/k3-ddrss/k3-ddrss.c : U_BOOT_DRIVER(k3_ddrss), k3_ddrss_probe()
- board/ti/j721e/evm.c : Board initialization
- Linux Device Tree under arch/arm64/boot/dts/ti/

## Important Function Flow

board_init_f()
 -> uclass_get_device(UCLASS_RAM,0,&dev)
 -> invokes k3_ddrss_probe()
 -> DDR controller initialization
 -> DDR timing programming
 -> DDR training
 -> Memory becomes available for later boot stages.

## Changes Performed

1. Built J721E EVM U-Boot.
2. Used AI-64 DDR configuration while generating tiboot3.bin.
3. Used AI-64 Linux Device Tree.
4. Tested multiple combinations of tiboot3.bin, sysfw.itb, tispl.bin and u-boot.img.
5. Verified boot progress using serial console.

## Final Working Combination

tiboot3.bin : EVM with AI-64 DDR configuration
sysfw.itb   : AI-64 compatible
tispl.bin   : EVM
u-boot.img  : EVM
Linux DTB   : AI-64
RootFS      : EVM

## Verification

- Linux booted successfully.
- /proc/device-tree/model reports BeagleBone AI-64.
- ~4 GB LPDDR4 detected.
- EVM userspace runs on AI-64.

## Key Conclusions

• DDR initialization is the critical board-specific step handled in tiboot3.bin.
• Linux requires the AI-64 Device Tree.
• EVM tispl.bin and u-boot.img can operate on AI-64 when DDR and DTB match the hardware.